| EVENT REPORT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. PROBLEM TITLE : Engine makes unusual noise when loaded and also gives steady white smoke from the exhaust |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date Occurred: |  |  | 2nd Sept, 2023 |  |  |  | Location: Adibawa Flow Station |  |  |  |  |  |  |  |  |  |
| Date Reported: |  |  | 2nd Sept, 2023 |  |  |  | Reported by: Field Operator |  |  |  |  |  |  |  |  |  |
| Event Type |  | Potential Threat (not yet occurred) Reliability/integrity – Trip Reliability/integrity – Equipment failure Reliability/integrity – Others |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Equipment Tag Number: NG. ADIA1.G-8201A-DRV |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sequence Of Event: - Field Operator started the Genset after a prolonged outage. - Field Operator reported unusual performance of the G3412A. - Joint Maintenance team who was at site too made efforts to understand the cause malfunction- and observed excessive white smoke from exhaust and unusual noise from one of the cylinder. - Team manually stopped the engine & carried out preliminary checks without success. - IA Workshop team was invited for further checks and issue resolution. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences: |  |  |  |  | Risk Assessment: (People, Asset, Environment, Community) |  |  |  |  |  |  |  |  |  |  |  |
| No deferment / outage Oil: bbl Gas: Water: Nil Flare: Nil Other: Replacement cost xxx USD Downtime: |  |  |  |  |  | A |  |  | B | C | D | E | Actual: A3 Potential: A4A |  |  |  |
|  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 3 | A |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |
| Immediate Corrective Actions Taken: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # | Immediate action |  |  |  |  |  |  | Date |  |  |  |  |  |  | By | Remarks |
| 1. | Manually Shutdown the Unit. |  |  |  |  |  |  | 2nd Sept, 2023 |  |  |  |  |  |  | Field operator |  |
| 2 | Troubleshoot cause of malfunction by Asset team |  |  |  |  |  |  | 2nd Sept, 2023 |  |  |  |  |  |  | Mechanical team |  |
| 3. | Raise Notification |  |  |  |  |  |  | 16th Sept, 2023 |  |  |  |  |  |  | CRO | 11995917 |
| 4. | Carry out 5-Why Causal Investigation |  |  |  |  |  |  | 22nd Sept, 2023 |  |  |  |  |  |  | Ojiabo Chiemezie |  |
| 5. | Replace failed components & Reinstate Unit |  |  |  |  |  |  | 20th Oct, 2023 |  |  |  |  |  |  | Mtce Exec / Asset |  |

| 2. WHAT DO WE THINK CAUSED THE PROBLEM? |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Investigation Team |  | Ojiabo Chiemezie, Emetubelem Muolee, Wlouchem Emmanuel, Omokhoa Harrison, Jiakpo Josephat, Inaibo Laiye, Okoro Fredrick, Elegbede Sakiru, Abass,K, Nkere,Rotimi Jaiola, Deresoma |  |  |  |  |
| Problem (Primary Effect) |  | - Expected: Genset G3412A to run and supply power to station without unusual noise - Actual: Genset G3412A made unusual noise while running - Impact: Power Outage to the facility as unit was manually shutdown |  |  |  |  |
|  |  | Why? / Immediate cause | Answer/Root Cause |  | 3. EVIDENCE? |  |
| Why 1 |  | Why was there unusual noise from Genset G3412A while in service? | Because a metallic component flickered within the combustion chamber |  | Unusual noise was witnessed within the station. Evidence of pitted Piston head |  |
| Why 2 |  | Why did a metallic component flicker within the combustion chamber? | - Because a component part of the valve failed 2.2 Loose contact within the external Rotating Parts 2.3 Failed Crankshaft Bearings 2.4 Misaligned Rocker arm assembly |  | Intrusive checks made by the workshop revealed a cut on the Cylinder 9 valve. No external loose contact No evidence of failed crankshaft bearing |  |
| Why 3 |  | Why did the component part of the valve fail? | 3.1 Age 3.2 Stuck Valve 3.3 Substandard valve Material 3.4 Wrongly installed Valve 3.5 Incomplete combustion 3.6 Prolonged Engine downtime |  | Provide evidence of run hours. (Runhours 707034) Mtce Plan include 3M. 12M & 24M |  |
| Why 4 |  | Why was there stuck exhaust valve? | 4.1Wet fuel gas (Wet charge) - Failed Spark Plug - Engine Under-utilisation - Lube oil into the combustion chamber - Insufficient Air intake - Excessive valve clearance opening - Engine prolonged outage |  | No stuck valve Healthy spark plug Smoke from exhaust did not suggest combustion chamber contamination with oil Failed engine exhaust valve Provide evidence of load at peak hours |  |
| Why 5 |  | Why was there engine prolonged outage? | 5.1 Unavailability of required component parts for repair |  |  |  |
| Why 6 |  | Why was the component part unavailable | Issues with Mantrac contract to supply material |  |  |  |
| Comments: |  |  |  |  |  |  |
| 4. WHAT SOLUTIONS DO WE HAVE IN MIND? |  |  |  |  |  |  |
| # | Proposed Action |  |  | Action Party |  | Target Date |
| 1 | Develop a preservative mtc plan for equipment not in use |  |  | Deresoma/Laiye |  |  |
| 3 | Verify adequate Mtc plan is in place |  |  | Abass k/Rotimi |  |  |
| 5. HOW WILL THE PROPOSED SOLUTIONS ELIMINATE THE CAUSES OF THE PROBLEM? |  |  |  |  |  |  |
| - This helps in lubricating moving components of the equipment, prevent seizure and make the equip ready for use |  |  |  |  |  |  |
| - Ensure that personnel responsible for operating and maintaining the engine are well-trained and know the significance of adhering to maintenance schedules. Prompt reporting of unusual noises or behaviors can lead to early intervention and prevention of more significant problems. |  |  |  |  |  |  |
| - Ensure minimum stock of critical spares for quick intervention |  |  |  |  |  |  |
| Incident Owner: Ogbuehi Charles |  |  |  |  |  |  |